Brake Adjustment for 6V-36

Reference HS-592 Head Assembly Drawing

Turn Power off to Saw

Remove Saw blade

Remove upper pulley

Push and hold brake reset lever up (located above start/stop button), to relieve tension on brake mechanism arm (located behind brake drum).

This allows removal of the brake pin (#7 on HS-592) after removing cotter pin and washer.

Brake reset lever may now be let down.

Brake adjustment is made by moving brake mechanism arm on splined shaft of brake assembly.

Caution! Before removing brake arm note exact position of brake arm on spline.

Loosen ½” bolt, remove brake arm and move 1 tooth only to the left (CCW). This will increase pressure on brake shoes when brake is actuated.

Retighten ½” bolt with brake arm back on shaft.

Holding brake reset lever up, reconnect brake link to brake arm with pin, cotter pin and washer.

Put pulley back on, increased brake pressure should be observed turning pulley.

Reinstall blade, crank up to proper tension and reset brake. Pulley should turn free.

Without saw running, test brake by removing blade tension until brake is actuated.