Operating Instructions

For

6V-36 Band Saw

Saw Blades:

Grob machines are not limited to certain types of saw blades.

Any type of blade from 1/8” to 1” wide and 242” to 248”

long can be used.  Open both doors and insert a saw blade.

By turning a crank located on the left hand side of the column

in a clockwise direction, which raises the saw pulley, tension

is applied to this blade. The tension indicator on the front of

the column will show when the proper tension is reached for

the width blade in the machine.  Rotate the saw pulleys manually

to check the tracking of the blade on the pulley.  By turning a

cantering knob on the right side of the head, the upper saw pulley

is tilted so that the band can track near the center of the pulley.

The saw blade guides are described on a separate sheet, explaining

their proper usage. See “Installing New Saw Band on Grob 6V-36 Band Saw” for details.

Limit Switches and Brakes:

There are 3 limit switches and 2 wheel brakes on this machine

provided for the safety of the operator.

The brake on the upper saw pulley is mechanically applied when a

blade breaks or when there is no tension on the blade.  When either

of these conditions exist a limit switch mounted on the brake

mechanism automatically opens, shutting off the machine.

The machine cannot be started until a new blade has been installed,

tension applied, and the brake reset.  Resetting of the brake is

accomplished by pushing a lever in the column upward as far as it

will go and then releasing it.  The machine can now be started.

The lower saw pulley and power unit are stopped automatically when

a blade breaks by a 50 ft/lb. Brake mounted on the drive motor.

This break may also be applied any time the operator desires to

stop the saw pulley and power unit quickly by pressing the mushroom

head “Main Stop” button.  However, under most conditions it is not

necessary to stop the saw so quickly, in which case the operator simply

presses the “Saw Stop” button which shuts off the motor but does not

apply the brake, allowing the motor to coast to a stop.

The limit switch on each of the doors prevent the starting of the machine

unless both doors are closed.

All 3 limit switches are connected in series, so that if one limit switch

is not working the machine will not run.

Shifting Gears and Changing Speeds:

The machine has a six speed gear box driven by a 3:1

ratio variable speed drive.  The shifting levers are located to

the left of the table and each can be moved into two positions

to obtain the six speeds of the gear box. The positions of these

levers are marked with the letters  “H-L”,  “O-I”, “O-I” which

coincide with the speed chart located on the lower left hand

side of the saw. Caution –Stop Motor Before Shifting Transmission.

Each speed of the gear box has a three to one range.  This speed

range is obtained by turning a crank located on the left side of

the column.  Changing of speed must be done only when machine

is running.   It is advisable to crank through the full speed range

daily to prevent the variable speed discs from becoming sticky

and insuring free movement.

Air Hydraulic Feed:

The air hydraulic feed unit consists of a surge tank, regulating valve,

operating valve and hydraulic cylinder connected to table feed

bars.  The surge tank is prefilled at factory with ISO-68 oil.

One quart in standard saw and one gallon in plate saw.  The

regulating valve is located in the lower left hand side of the

machine.  It can be adjusted to provide a feed pressure from 0

to 300 lbs.  The amount of feed pressure is shown on a gauge

directly above the valve.  The hydraulic cylinder is double acting,

with oil pressure doing the feeding and air pressure doing the

rapid returning.  The operating valve controls the cycling of the

air hydraulic feed.

For Feeding:

Push operating valve handle forward.  This allows air to enter top

of surge tank forcing oil into hydraulic cylinder and exhausting air

from the cylinder through valve.

For Returning:

Pull valve handle out.  This allows air to enter hydraulic cylinder

and the air from the top of the surge tank to exhaust through valve.

Placing valve handle in the middle position will stop the unit.  If

the table feed has a jerking motion during the feeding cycle, it may

be caused by air in the oil line or insufficient oil in the surge tank.

Air in the oil line can be removed by cycling the table feed a few times.

If this does not stop the jerking motion, fill oil in the surge tank and

repeat above procedure.  The air inlet for this unit is in the lower left

hand side of the base.

To tilt the table, the three table clamp nuts must be loosened and the table

can be moved by turning the table tilt crank.  When the proper setting is

reached, retighten the three table clamps.

Saw Blade Guides:

Saw blade guides are available for different types of cutting.

- Single Piece Guides for 1/8” - 1”  Blades

The single piece guides available in Hard Aluminum

# Bronze are self seating.   This self-seating quality

provides a large bearing area for the back of the blade

which eliminates “mushrooming” on the surface.  This

prevents pinching in the cut and lengthens the life of

the blade. This permits heavier pressure to the saw blade,

thereby providing faster cutting.  The compact size permits

close support immediately above and below the work.

Quick and easy change for various saw blades is also

a benefit.  Single Piece Guides are also available in

Tungsten Carbide for longer guide life.

- Three Piece Guides for 1/8” - 1”  Blades

The three piece guides are available in Hard Aluminum     Bronze or Tungsten Carbide. They have 2 adjustable side pieces and a back piece for each holder.  The adjustable side pieces insure prolonged accurate guiding and allow for various blade thickness.

Contour cutting is made easy with the adjustable side pieces.

- Roller Guides for 5/8” - 1” Blades

The roller guides consist of three rollers, 4” in diameter.

There are two side rollers, one of which is adjustable and

one adjustable rear roller.  The rollers are ball bearing mounted.

The rollers are adjustable to maintain proper support and

backing for the blade, which is very essential in making accurate cuts and prolonging the life of the saw blade.

Rollers do not provide good side guiding for accurate

contouring.

Side rollers are available for 1/4”,  3/8” and 1/2” blades and are available upon request.

Lubricating Instructions:

Before running the machine, fill the transmission with

4 ½  gallons of  85w-140 gear oil or equivalent.

A fill plug located behind the table is provided for this purpose.

This oil should be changed once a year.

Plate Saw Table Feed: WARNING! Remove air supply to saw before proceeding. With table all the way out towards operator

(cylinder piston rod completely into cylinder) oil level is even

with the fill hole on the reservoir.

For maintenance of the Gerbing Roto-Cone unit, we enclose

their lubricating instructions which should be followed closely.

On the upper saw pulley there is a grease chamber for

the ball bearings which should be repacked every 2,000 hours.

Remove both plugs to repack chamber.

The felt pad on the upper saw guide bar should be

saturated with lubricating oil at regular intervals.

If difficulties are encountered on specific sawing jobs,

we suggest contacting a field man from any of the saw blade

manufacturers or send test samples to GROB INC.  We will

make recommendations as to proper blade, speed, and feed,

at no cost to the customer.

GROB INC.

GRAFTON,  WI    53024